Bio-SCREW (Shafted)

Tag Number/Serial Number/Customer Provided #

Operations and Maintenance Manual

PLANT NAME

LOCATION

Specification Section XX XX XX

Engineer of Record: XXXX

Revision X

MONTH YEAR

Table of Contents

Contact Information	4

1.	Scope of Supply	5

1.1	Equipment Data	5

1.2	Special Tools	5

1.3	Field Service	5

1.4	Scope Exclusions	5

1.5	Spare Parts	5

2.	Safety Precautions	6

2.1	Equipment-Specific Safety	6

3.	Delivery, Handling, and Storage	8

3.1	Upon Equipment Arrival	8

3.2	General Handling and Storage Information	8

3.2.1	Avoiding Cross-Contamination Between Materials	9

3.3	Storage	10

3.3.1	General Requirements for Indoor Storage	11

3.3.2	General Requirements for Outdoor Storage	12

3.3.3	Gear Units	12

3.3.4	Electric Motors and Actuators	13

4.	Installation Instructions	15

4.1	Before Installation	15

4.2	Detailed Installation Instructions	17

5.	Operating Procedures	21

5.1	General Equipment Operation	21

5.2	Initial Startup	21

5.3	Startup	22

5.4	Shutdown	22

5.5	Shutdown (Alarm Condition)	23

6.	Maintenance, Lubrication, & Troubleshooting	25

6.1	Preventative Maintenance	25

6.2	Corrective Maintenance	27

6.3	Lubrication	27

6.4	Troubleshooting	32

7.	Policies	35

7.1	Warranty	35

7.2	Liability	35

7.3	Returns	35

7.4	Delivery	35

7.5	Thread Lubricant Information	36

7.6	Standard Erecting Practices	36

Table of Figures

Figure 1. Breather Valve with Protective Band (in Black)	19

Figure 2. SEW Eurodrive Mounting Position "XXX" and Location of Lubrication Components	28

Figure 3. Oil Level Plug (Painted Red) and Breather Valve (Protective Band Removed) (FOR REFERENCE ONLY)	28

Figure 4. Example Sight Glass in Lieu of Oil Level Plug (If Applicable) (FOR REFERENCE ONLY)	29

Figure 5. SEW Eurodrive Diagram for Checking Oil Level	31

List of Tables

Table 1. Typical Bolt Torque Chart for Stainless Steel and Non-Ferrous Fasteners	16

Table 2. Bio-SCREW Preventative Maintenance	25

Table 3. Bio-SCREW Component Lubrication Requirements	29

Table 4. Bio-SCREW Troubleshooting	32

List of Appendices

Appendix A: Catalog Cuts

Appendix B: Equipment Drawings

Appendix C: Calculations

Appendix D: Startup Reports

Appendix E: Controls

Appendix F: O&M Revision History

Local JMS Representative:

Rep Name

Company Name

Company Address

City, State Zip

(XXX)XXX-XXXX

JMS Project Manager:

PM Name Here

Jim Myers & Sons, Inc.

PO Box 38778

Charlotte, NC 28278

Phone: (704)554-8397

Fax: (704)554-9113

Contact Information

For general inquiries regarding field services, contact custsvc@jmsequipment.com.

For general inquiries regarding aftermarket parts, contact AMP@jmsequipment.com.

# Scope of Supply

The JMS scope of supply includes X (Number) X (description from scope), also referred to herein as (a) “Bio-SCREW”(s). INSERT OTHER DETAILS ABOUT THE BIO-SCREWS FROM SCOPE HERE

The Bio-SCREW(s) is/are supplied with the following features:

- INSERT SCOPE OF SUPPLY FROM APPROVED SUBMITTAL, including everything not already mentioned in sentence above

## Equipment Data

Please refer to the above Scope of Supply, the notes on the drawings included in Appendix B: Equipment Drawings, and the calculations included in Appendix C: Calculations for detailed Bio-SCREW equipment parameters.

## Special Tools

While no special tools are required for working with the Bio-SCREW, care should be taken to avoid cross-contamination between carbon steel and stainless-steel components when installing, maintaining, or otherwise working with the equipment or components. Please refer to the additional information about carbon steel contamination provided in Section 3.2.1 - Avoiding Cross-Contamination Between Materials.

## Field Service

The JMS scope of supply xxxxx INCLUDE FROM SCOPE XXX

Additional service trips and service days are available at an additional cost. Compensation is required for any additional visits caused by site delays that result in the inability to complete field services during scheduled site visits.

Before beginning equipment installation, schedule JMS field service site visits in advance using the contact information provided in this O&M manual. Site visits should be scheduled at least four weeks in advance.

## Scope Exclusions

The following items are excluded from the JMS scope of supply:

- XXXXX INCLUDE FROM SCOPE XXX

## Spare Parts

The JMS scope of supply includes the following spare parts. Please note that a complete table of spare parts is also included on the GA drawings.

- Insert as necessary, otherwise can simply say spare parts aren’t included for this project and remove all this text

# Safety Precautions

Safety must always be considered a basic factor in machinery operation. Most accidents are the result of carelessness or negligence. The following safety instructions are basic guidelines and should be considered as minimum provisions. Please note that not all precautions are applicable to all equipment components. Additional information should be obtained by the purchaser from other sources, including the latest editions of American Society of Mechanical Engineers Standards ANSI B20.1, ANSI B11.0, and ANSI A1264.1, as well as ANSI/ASSP Z244.1-2016.

It is the responsibility of the Contractor, installer, Owner, and user to install, maintain, and operate the equipment components and assemblies manufactured and supplied by JMS in such a manner as to comply with the Williams-Steiger Occupational Safety and Health Act. Furthermore, all parties should comply with all state and local laws and ordinances, as well as the American National Standards Institute Safety Code.

It is extremely important to follow the below precautions to prevent personal injury or property damage:

- Only persons properly trained and familiar with JMS equipment should be permitted to operate or maintain the unit.

- LOCK OUT/TAG OUT ALL POWER prior to any inspection or maintenance.

- Periodically run the equipment for a few minutes to check for excessive vibration, loose fasteners, security of covers and guards, noise, and bearing and gearmotor temperature.

- ALWAYS operate the equipment with covers, guards, and safety labels in place.

- NEVER walk on or across equipment covers, guards, or grating.

- DO NOT place hands, feet, or clothing in equipment openings.

- DO NOT poke or prod the equipment or material in the equipment.

- ALWAYS practice good housekeeping to maintain a clear view of the equipment and all safety devices.

## Equipment-Specific Safety

Please follow all local safety regulations when working around the Bio-SCREW. Additional safety requirements are outlined below.

- Follow all plant and industry LOCK OUT/TAG OUT (LOTO) procedures and practices throughout the handling, installation, commissioning, startup, and maintenance of the Bio-SCREW. This protects installation personnel from the unexpected release of all forms of potential energy. Equipment should be locked out in such a manner that the equipment cannot be restarted by anyone, including employees that are remote from the area. Other secondary devices should not be used as a substitute for locking out equipment power.

- Follow all plant and industry confined space procedures and practices throughout the handling, installation, commissioning, and startup of the Bio-SCREW.

- Observe all posted safety labels while working on or around the Bio-SCREW.

- The exposed screw and moving parts can cause severe injury; follow all plant and industry LOTO procedures and practices while working on or around the Bio-SCREW and its components, including prior to the removal of any guards, railings, or covers for inspection or service.

- Operate the Bio-SCREW only when all approved covers, railings, and guards are in place.

- Keep clothing, body parts, and hair away from the screws.

- Block any gravity loads prior to performing inspections or maintenance procedures.

- Clean up spillage or adhered solids near or on moving parts only when the equipment is locked out and all necessary covers, railings, and guards are in place.

- Do not modify the controls unless explicitly directed.

- Verify that all controls are visible and accessible at all times.

- Report any unsafe conditions.

- Do not use the Bio-SCREW or any of its components for a purpose other than that for which it was intended. Do not attempt field modification of the Bio-SCREW or any of its components.

- Do not poke or prod material into the Bio-SCREW or any of its components via a bar or stick inserted into its openings during normal operation. Do not attempt to clear a jammed Bio-SCREW until the equipment has undergone appropriate LOTO procedures.

- Do not overload the Bio-SCREW or otherwise exceed the equipment speed, capacity, volume, material density, or rate of flow for which the Bio-SCREW and gearmotor were designed.

- Do not utilize the Bio-SCREW for any application other than that for which it was designed.

# Delivery, Handling, and Storage

## Upon Equipment Arrival

Although all possible precautions are taken to protect equipment against damage or losses during shipment, before accepting the shipment, check all items against the packing list for shortages and inspect for evidence of physical damage. In either case, immediately notify JMS using the contact information included herein. Keep a record of all claims and correspondence. Photographs are recommended.

As detailed and agreed upon in the original contract and purchase order, all equipment and components delivered to the receiving location should be inspected immediately. A full and complete inspection report noting any visible damage must be transmitted to JMS within ten (10) days of the delivery date. If a written report is not received by JMS within this period, it will be conclusively assumed that the equipment was received in good condition and meets the requirements of the purchase order and/or equipment specification. Failure to submit a full and complete inspection report in the time constraints dictated herein constitutes unqualified acceptance by the buyer or customer, and the buyer or customer waives any rights to rejection or remediation of the delivered equipment.

Do not remove protective covers unless there are indications of damage. Boxes opened for inspection and inventory should be carefully repackaged to ensure protection of the contents, or the parts should be stored in a safe place. Refer to Section 3.3 - Storage. Examine all packing boxes, wrappings, and covers for items that may be attached to them, especially if these items are to be discarded.

## General Handling and Storage Information

Upon completion of the initial inspection, the equipment should be moved from the delivery truck to temporary storage (refer to Section 3.3 - Storage) or immediately installed (refer to Section 4 - Installation Instructions). When maneuvering the equipment, go slowly and use extreme caution. Watch for accidental damage caused during handling and document any issues. It is the responsibility of the handler and installer to ensure that appropriate measures are taken to prevent damage to the equipment during handling and installation.

Provided in the following sections are instructions for handling and storing the equipment. These instructions are intended to cover the basics but will not cover every conceivable situation. Use good judgment when handling and working with the equipment.

Unless otherwise noted, lifting lugs or spreader bars with slings are the recommended support method for lifting all equipment in instances where equipment components cannot be moved manually due to weight or other constraints. When utilized, the unsupported span of spreader bars should be no longer than 12 feet. Do not attempt to lift any equipment from only one support point. When support locations or lifting lugs are not specified on the drawings, support locations must be carefully selected with consideration to the overall equipment weight distribution (i.e. consider the positioning of unusually heavy items such as gearmotors or gates). Support points should be selected such that optimal load balance is achieved to avoid bending or other damage to the equipment.

Please refer and adhere to any additional best practices for screw conveyors as detailed in all applicable CEMA standards, including ANSI/CEMA Standard No. 350 “Screw Conveyor for Bulk Materials” and ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual”.

    - Avoiding Cross-Contamination Between Materials

This equipment is partially or wholly constructed from corrosion-resistant stainless steel. The Owner and/or installer must protect the equipment against damage to and/or contamination of the stainless steel’s protective oxide layer by taking all necessary precautions to prevent stainless steel equipment from coming in contact with any carbon steel or carbon steel dust throughout the handling, installation, commissioning, and startup of the system. Failure to protect stainless steel equipment from damage to and/or contamination of its protective oxide layer will void the manufacturer’s material warranty.

As a general practice, keep carbon steel components separate from stainless steel components. Avoid stacking components in a manner such that carbon steel and stainless steel will be in contact. While not an exhaustive list, protect stainless steel equipment from the following forms of carbon steel:

- Truck beds

- Forklifts

- Cables

- Chains

- Tools

- Wire brushes

- Grinding wheels

- Grinding dust

- Nylon lifting straps

If work on any carbon steel components is being performed on site near the storage area for this project, cover stainless steel components with tarps or paper to protect them from coming in contact with carbon dust particles. If work is being performed indoors, fans should be used to keep carbon steel dust from falling onto stainless steel components. If work is being performed outdoors, position the work involving carbon steel such that it is located downwind of the stainless steel component storage area.

Select tools for installation and maintenance of the stainless steel equipment that are stainless or non-magnetic. Tools made of aluminum bronze, chrome alloys, or stainless alloys are examples of acceptable tool materials. Ensure the tools selected meet the torque requirements for the equipment as detailed on Table 1 in Section 4.1 - Before Installation and the drawings.

Lifting straps previously used for handling carbon steel should not be used to handle stainless steel equipment components. If there is no method for tracking which material the lifting straps may have handled previously, new lifting straps should be used.

Because a thin layer of chrome oxide naturally forms on the surface of stainless steel, stainless steel resists corrosion. During welding and machining operations, the chrome oxide layer can be damaged. To rebuild this corrosion-resistant layer, pickling and passivation in accordance with ASTM A380 and A967 are conducted on necessary parts of the equipment before it leaves JMS. However, if the corrosion-resistant layer is further breached by installation and/or handling practices, iron particles may be introduced. The introduction of these particles may lead to the appearance of surface corrosion on the stainless steel component. In the majority of JMS equipment, the surface corrosion is merely aesthetic in nature and does not have any impact on equipment performance. However, these issues can be mitigated or prevented entirely during installation by following the guidance included in this O&M manual.

Some surface corrosion is practically unavoidable over time for most plants due to the environment in which the equipment will be used. Chemicals such as chlorine can introduce corrosion to stainless steel components without the presence of iron particles, and a damp environment can propagate surface corrosion over time. Most corrosion does not have a performance impact on the equipment over its operational life assuming suitable performance characteristics have been specified. Contact JMS using the contact information provided in this O&M manual if concerns arise about the presence of surface corrosion on JMS equipment.

If surface corrosion is noted on stainless steel and needs to be removed, perform the following steps:

- Clean the affected area thoroughly with a clean rag to remove any moisture on the surface.

- Remove the corrosion from the contaminated area with an abrasive pad that is appropriate for working with stainless steel.

- Treat the abraded area with a local passivation chemical. JMS recommends the use of CitriSurf 2310.

## Storage

Please note that the equipment storage instructions provided herein are the JMS standard requirements for all equipment and cover a variety of components comprehensively. Some instructions may not apply to the equipment covered in this O&M manual. Please disregard any instructions that clearly do not apply. In the case where a vendor O&M manual has been attached to this O&M manual and includes more stringent and/or differing equipment storage requirements, please adhere to the vendor O&M manual instructions. Failure to comply with these equipment storage instructions may void any and all warranties for the equipment.

After the equipment has been checked for any transit damage, JMS recommends the following procedures for equipment storage. Storage refers to the period of time following shipment but prior to installation in which the equipment or component must be protected from damage. For the purposes of this O&M manual, the following requirements are intended to apply when equipment is stored for 30 days or longer on site prior to installation. Requirements may vary based on the duration of the equipment storage.

It is preferred that all equipment be warehoused to protect against inclement weather, dust, and/or damage by vehicles. Similarly, all spare parts should be warehoused until ready for use. Storage inside a building is preferred but covering the equipment or component with a canvas tarp is acceptable unless otherwise indicated. Do not cover the equipment or component with a polyethylene sheet, as this will cause and trap condensation. Additional recommendations for indoor and outdoor storage of various components are highlighted below.

Please refer and adhere to any additional best practices for screw conveyors as detailed in all applicable CEMA standards, including ANSI/CEMA Standard No. 350 “Screw Conveyor for Bulk Materials” and ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual”.

### General Requirements for Indoor Storage

The following general requirements are applicable to all equipment or components stored indoors.

- Store the components in a cool, dry area that is not subject to extreme temperature fluctuations. Protect components from heat and flame, as well as from dirt, mud, and flying debris.

- Ensure the area for storage has adequate ventilation.

- Ensure the storage area ground surface is level.

- If the system includes a Mega-SETTLER, support the plate packs with dunnage in the form of 6” x 6” wood beams to raise them off of the ground and prevent distortion. Always support the plate packs under the vertical uprights in the plate settler truss frame. The individual plates inside the plate pack frame should not under any circumstance contact any part of the dunnage or ground.

- Provide space heaters for heating and drying.

- Provide adequate support under all equipment to prevent distortion and to raise equipment above any undesirable ground or floor conditions. Ensure heavy objects are not stacked on top of the component.

- Ensure tenting and/or tarpaulins (if any) are well supported to provide sufficient ventilation.

- Re-coat parts which may have been scratched or wiped clean of their protective coating in shipment with paint, primer, or grease.

- Shield painted surfaces, including bitumastic coatings, from direct sunlight, which can lead to warping, damage, or chipping.

- Check the equipment periodically for accumulations of moisture and/or surface corrosion and remove. If surface corrosion is present, prepare the surface and re-coat with the proper material. Increase ventilation and reduce humidity.

- In equipment containing rotating parts, rotate the equipment one and a half (1.5) rotations every two weeks. Rotation can be performed by bumping the motor (if applicable) or disengaging the motor (if applicable) and rotating by hand.

### General Requirements for Outdoor Storage

The following general requirements are applicable to all equipment or components stored outdoors. It is the responsibility of the Contractor to protect JMS equipment from damage to equipment due to outside storage.

- If warehousing is not possible, choose a site that has adequate drainage, ventilation, and size to maintain space for air circulation and access by inspecting personnel.

- Shield the materials from direct sunlight, rain, snow, dust, etc. with tenting if possible or well-supported tarpaulins at a minimum. Ensure good cover is provided at all times. Ensure that the equipment is protected from contact with other machinery, equipment, etc.

- Provide space heaters for heating and drying in cold or wet weather.

- Use blocks to raise all material off the ground a minimum of 6” higher than any expected level of rain, snow, etc. Components should be stored a minimum of 6” off the ground. If the system includes a Mega-SETTLER, the use of 6” x 6” wood beams is recommended to support and raise the plate packs a minimum of 6” higher than any expected level of rain, snow, etc. Always support the plate packs under the vertical uprights in the plate settler truss frame. The individual plates inside the plate pack frame should not under any circumstance contact any part of the dunnage or ground.

- Provide adequate support of all fabricated steelwork to prevent warping.

- Perform frequent checks to locate surfaces affected by corrosion and accumulations of condensation. It is the responsibility of the Contractor to protect JMS equipment from surface corrosion due to outside storage, including corrosion occurring after the equipment is delivered to the job site but before final finish painting. If surface corrosion is found on painted surfaces, remove the surface corrosion, re-prime, and re-paint the surface. If the surface was originally grease-coated, remove the surface corrosion and re-coat the surface with waterproof grease.

- Block any open inspection doors, etc. slightly to increase ventilation in housings.

- In equipment containing rotating parts, rotate the equipment one and a half (1.5) rotations every two weeks. Rotation can be performed by bumping the motor (if applicable) or disengaging the motor (if applicable) and rotating by hand.

### Gear Units

In addition to the general requirements, adhere to the following additional requirements for storage of gear units.

- Although some components are shipped dry, units generally include run-in oils containing surface corrosion-proofing additives. The additives will protect internal gearing for an extended period (see subsequent bullets) under good storage conditions. Verify that the unit is prepared for extended storage with the equipment manufacturer.

- The gear units should remain tightly sealed until startup.

- Store gear units according to the mounting position.

- If the gear unit is packed in containers and shipped with desiccant and a moisture indicator sealed in the plastic wrap, regularly inspect the packaging and humidity indicator. Relative humidity should be less than 50% at all times. The gear unit should be stored under a roof and protected against rain and snow. The unit can be stored for up to 3 years.

- If the gear unit is received open (no packaging), perform regular inspections. Check for cleanness and mechanical damage and inspect the corrosion protection. The gear unit should be stored under a roof and enclosed at a constant temperature and atmospheric humidity (between 41 and 122 degrees F; less than 50% relative humidity). Ensure there are no sudden temperature variations, aggressive vapors, or shocks, and control ventilation with a filter free from dust and dirt. The unit can be stored for 2 years or more.

### Electric Motors and Actuators

In addition to the general requirements, adhere to the following additional requirements for storage of electric motors and actuators.

- Do not remove electrical compartment covers unless specifically instructed in the vendor O&M manual.

- Block open control panel covers or doors so air can circulate.

- Open all cartons and remove all sealing films unless the carton specifically states otherwise. Remove moisture-absorbing materials if found in the cartons. Remove any plastic shock-absorbing particles surrounding the cartons and/or films.

- Store small apparatuses in the original containers (if received dry).

- If not mounted, store motors in the original containers in a clean, dry, and protected warehouse.

- Ensure the storage area is free from vibration and extreme fluctuations in temperature.

- Ensure motors are fully greased.

- Ensure all drains remain fully operable while in storage, and/or remove the drain plugs. The motors must be stored so that the drain is at the lowest point. All breathers and automatic “T” drains must be operable to allow breathing at locations other than through the bearing fits. Motors should be stored in their proper mounting position.

- Where motors are not in the original containers but are instead mounted on other pieces of machinery, the mounting must be such that the drains and breathers are fully operable. The drains must be kept at the lowest point on the motor and/or the drain plugs removed so that all condensation can drain freely.

- Apply corrosion resistant coating to external motor parts when the motor is stored in a corrosive environment.

- Coat all machined parts with Cosmoline (or similar material) if not already coated.

- Connect all units equipped with heaters (if storage conditions in any way simulate or approach operational conditions).

- Keep the motors warm. If the motors are equipped with space heaters, energize them at all times. If space heaters were not supplied, auxiliary heat must be used to keep the motor windings warm and free of condensation.

- If the motor or actuator is installed but not cabled, it is recommended that any plastic transit cable entry plugs are replaced with metal plugs (which are sealed with PTFE tape).

- Check windings for resistance with a megohm meter at the time equipment is put in storage. At termination of storage, repeat this resistance check prior to applying power. The minimum reading should be 1 megohm for motors rated 600 volts or less. Any drop below this point indicates the need for electrical or mechanical drying. When storing a large quantity of motors, perform an inspection of a sample size of the motors by removing the end brackets and visually inspecting for the presence of water in the grease or surface corrosion on the bearing. If water or surface corrosion is present, replace the bearings and re-lubricate.

- Purge grease in the motors at the time of removal from storage, ensuring that an ample supply of fresh grease is in each grease cavity.

- If control panels are provided, adhere to the following additional storage recommendations:

  - If the control panel does not arrive to site wrapped in plastic, wrap the control panel in a covering of heavy-duty plastic or similar material to help prevent the entry of dirt and dust.

  - For control panels not installed and energized immediately, store in a clean, dry place. Maintain a storage temperature between -30 and 65 C (-22 and 149 F). If the storage temperature fluctuates or humidity exceeds 60%, use a space heater to help prevent condensation. If possible, store the control panels in a heated building that offers adequate air circulation and protection from dirt and water.

  - Control panels that are designed for indoor applications do not have sufficient packaging for outdoor storage. If these control panels are to be stored outdoors, install temporary electrical heating to help prevent condensation. Add packaging for protection from the outside elements. A space heater that is rated at 200 watts per section is adequate for the average control panel. Remove all loose packaging and flammable materials before you energize space heaters.

  - Keep dry internally all unenergized control panels for outdoor use by installing temporary heating or energizing optional self-contained space heaters.

# Installation Instructions

## Before Installation

Read the entirety of the provided installation instructions[, including information on stainless-steel hardware and the anchor manufacturer’s installation instructions provided in Appendix A: Catalog Cuts], before proceeding with installation. Examine field conditions as early as possible and take field dimensions to confirm that equipment as designed can be installed according to these instructions. Notify JMS immediately if there are undocumented field conditions which are adverse to successful equipment installation. The Contractor must also inspect the latest “APPROVED” or “RELEASED TO FAB” plan and elevation drawings prior to and throughout the installation process.

The following erection and installation procedures are meant to serve as guidelines for installing the JMS equipment and may not address site- or project-specific issues.

If any fit-up problems with equipment are noted during installation, alert JMS immediately using the contact information provided herein. Do not rework any equipment without contacting JMS. Any unauthorized rework will void the warranty on the equipment. JMS reserves the right to have its own personnel or contracted vendors correct field problems. Problems other than minor misfits as defined by AISC 303 should be communicated in writing to JMS using the contact information provided in this O&M manual via the project’s non-conformance reporting procedure.

Throughout the installation procedures described herein, proper industry and plant LOCK OUT/TAG OUT procedures (and confined space entry procedures as necessary) must be followed. These actions protect personnel against electrical and/or mechanical hazards and environments, including asphyxiants.

Please note that all figures depicting the equipment included throughout this O&M manual are for illustrative purposes only and may not reflect the exact conditions or equipment provided for this project. Refer to the drawings for detailed and specific equipment information.

While no special tools are required for working with the Bio-SCREW, care should be taken to avoid cross-contamination between carbon steel and stainless-steel components when installing, maintaining, or otherwise working with the equipment or components. Please refer to the additional information about carbon steel contamination provided in Section 3.2.1 - Avoiding Cross-Contamination Between Materials.

Before beginning equipment installation, schedule JMS field service site visits in advance using the contact information provided in this O&M manual. Refer to Section 1 - Scope of Supply for detailed information about what field services are included. Site visits should be scheduled at least four weeks in advance.

Utilize the bolt torque chart provided in Table 1 throughout installation as necessary for general bolt torque values. Do not use impact tools to torque the bolts. Refer to the drawings for unique torque values that may differ from what is provided in Table 1. Please also note that Table 1 does not apply to anchors[; refer to the anchor manufacturer’s installation instructions provided in Appendix A: Catalog Cuts for details on anchorage installation requirements]. For 304 stainless-steel hardware, refer to the column labeled “18-8 and 316 Stainless Steel”.

Additionally, refer to the Bolt Torque Chart provided in ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual” for information not provided in this Table 1 as needed.

Table 1. Typical Bolt Torque Chart for Stainless Steel and Non-Ferrous Fasteners

Please refer and adhere to any additional best practices for screw conveyors as detailed in all applicable CEMA standards, including ANSI/CEMA Standard No. 350 “Screw Conveyor for Bulk Materials” and ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual”.

## Detailed Installation Instructions

- Prepare Components for Assembly

  - Remove any strapping, wire, or other tethering devices that may have been used during transit from the Bio-SCREW.

  - Gather all loose components and hardware.

- Support Installation

  - Inspect the proposed mounting surface(s) for the Bio-SCREW support legs for levelness. The mounting surface must be level and true to avoid unnecessary bending or distortions in the Bio-SCREW. If the mounting surface is not level, make necessary provisions (such as shims or grout) to ensure that the Bio-SCREW will be installed level and straight. Periodically check for horizontal (and vertical if Bio-SCREW is not inclined) levelness and straightness in the Bio-SCREW throughout installation.

  - If precast anchors are used, ensure that the Bio-SCREW supports are placed onto the existing anchors as detailed on the drawings and loosely install the Bio-SCREW supports using the provided hardware.

  - If non-precast floor-mounted anchors are used:

    - Install the anchors and associated hardware as shown on the drawings and as detailed in the anchor manufacturer’s installation instructions included in Appendix A: Catalog Cuts. Ensure that the appropriate cure time is observed.

    - After the cure time has been observed, carefully position the Bio-SCREW supports onto the anchors and loosely fasten the Bio-SCREW supports onto the anchors via the provided hardware.

  - If non-precast overhead anchors are used:

    - Install the Bio-SCREW supports to the overhead mounting surface as detailed on the drawings using the provided hardware.

  - Install any additional support bracing using the provided hardware as detailed on the drawings.

  - Tighten all hardware according to the directions provided in Section 4.1 - Before Installation if not otherwise specified in the manufacturer’s recommendations included in Appendix A: Catalog Cuts (if any).

- Bio-SCREW Installation

  - [Utilize the drawings to position the trough segments in the appropriate order and direction, loosely installing them onto the supports and to each other using the provided hardware. Matchmarks have been provided on the trough segments as appropriate for ease of assembly.

  - Utilize piano wire or equivalent to align the trough bottom centerlines carefully and precisely. Verify that the trough centerline and piano wire are a consistent distance apart (within +/-1/8”) across the length of the trough. If needed, refer to ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual” for detailed information about trough alignment.

  - Fully tighten the trough segments to one another. Refer to Section 4.1 - Before Installation for details regarding hardware tightening. Verify that the gasket is securely fastened between the trough segment mating surfaces.

  - Fully tighten the trough segments onto the supports. Refer to Section 4.1 - Before Installation for details regarding hardware tightening.

  - Confirm trough alignment once more using piano wire or equivalent.

  - Refer to the drawings (included in Appendix B: Equipment Drawings) to determine the correct screw flight orientation and position the screw segments inside the trough appropriately. Matchmarks have been provided on the screw segments as appropriate for ease of assembly.

  - Insert the shaft coupling into the two adjacent screw ends and bolt the screw segments together using the provided hardware. Note that all coupling bolts should be tightened to the proper torque rating; it is recommended that all coupling bolts be tightened to only 75% of the torque values prescribed in previous Table 1 and greater Section 4.1 - Before Installation to avoid overtightening and potential failure in tension.

    - If hanger bearings are utilized, slide the hanger bearing over the shaft coupling prior to attaching the coupling to the second screw segment and bolt the hanger bearing to the trough or support bracket as detailed on the drawings.]

  - [Position the Bio-SCREW on the supports in the orientation detailed on the drawings. Verify that the Bio-SCREW is only contacting the supports at the designated support connection locations as detailed on the drawings.

  - Attach the Bio-SCREW to the supports using the provided hardware. Tighten the hardware according to the directions provided in Section 4.1 - Before Installation, verifying that the Bio-SCREW is secure and level upon completion.]

- Gearmotor Wiring and Inspection

  - Wire the gearmotor in accordance with the manufacturer’s installation instructions provided in Appendix A: Catalog Cuts.

  - After electrical connections have been made and before processing any material, turn the Bio-SCREW on and inspect the screw for proper direction of travel as detailed on the drawings. Incorrect screw rotation can result in serious damage to the Bio-SCREW, gearmotor, and associated upstream and downstream equipment. If screw rotation is incorrect, utilize a certified electrician to reconnect electrical leads and reverse gearmotor rotation.

  - Gearmotors are supplied with the correct grade of lubricant for the specified mounting position prior to arriving at site; however, it is important to verify that the lubrication is at the appropriate level prior to operating the gearmotor. Refer to Section 6.3 - Lubrication for additional information. Consult with JMS using the contact information provided in this O&M manual prior to operating the gearmotor if there is concern regarding the lubricant level.

  - Remove the breather valve’s protective band on the gearmotor if present. Refer to Figure 1 for an example of a protective band on a breather valve (protective band in black). Note that the figure is representative only and may not directly reflect the orientation of the motor or breather valve provided for this project. If the protective band has already been removed, no action is needed.

Figure 1. Breather Valve with Protective Band (in Black)

- Ancillary Component Installation and Adjustment

  - E-Stop Switch and Cabling Installation

    - If the E-Stop switch was provided separately, install the E-Stop switch at the mounting location shown on the drawings. A bracket has been mounted to the Bio-SCREW for this purpose.

    - Route the E-Stop cabling through the pre-installed brackets along the length of the Bio-SCREW.

    - Refer to the manufacturer’s installation instructions for the E-Stop provided in Appendix A: Catalog Cuts for additional information, including wiring information.

  - Zero-Speed Switch Installation

    - Install the zero-speed switch at the mounting location shown on the drawings.

    - Refer to the manufacturer’s installation instructions for the zero-speed switch provided in Appendix A: Catalog Cuts for additional information, including wiring information.

- Final Inspection

  - Inspect bearings and seals (if seals are not integral to the gearmotor) for possible misalignment which may have occurred during shipment. If seals are not integral to the gearmotor, verify that the Bio-SCREW shaft is centered in the end seals. Realign if necessary.

  - Verify that all bearings are lubricated according to the information provided in Section 6.3 - Lubrication.

  - Verify that all safety labels are firmly affixed to the Bio-SCREW and visible.

  - Before processing any material, turn on the Bio-SCREW gearmotor and inspect the freedom of rotation of the entire screw using a dial indicator or similar tool. Total shaft indicator runout at the bearing should not be greater than 0.030 inches. If runout is greater than 0.030 inches, straighten the bent screw assembly. Consult JMS personnel using the contact information provided in this O&M manual if straightening the screw assembly does not correct the problem.

  - Verify that the entire Bio-SCREW assembly is horizontally (and vertically unless the Bio-SCREW is inclined) level and straight. Adjust the anchorage bolts or connecting hardware as required to achieve this.

  - [Grout beneath the supports using non-shrink grout as detailed on the drawings.]

  - Rotate the Bio-SCREW by hand to ensure that no binding occurs before proceeding with initial startup (detailed in Section 5.2 - Initial Startup). If binding occurs, find and report the source of the binding to determine appropriate mitigative measures.

  - Install the Bio-SCREW cover (if not already installed) by performing the following steps:

    - Position the provided gasket on the trough flange where it will mate with the cover.

    - Position the Bio-SCREW covers on the trough, referring to the drawings to determine proper cover sequence and positioning of inlet openings. Handle the covers carefully to avoid warping and bending.

    - Install the Bio-SCREW covers using the provided fasteners. Refer to Section 4.1 - Before Installation for details on fastener tightening. Do not tighten the covers excessively as doing so may warp the cover and result in leakage.

# Operating Procedures

## General Equipment Operation

The JMS shafted Bio-SCREW is used to smoothly and continuously transfer or convey bulk material from one step in the plant process to the next. With a wide variety of uses in water and wastewater industries, the Bio-SCREW consists of one or more inlet locations to receive material from upstream feed equipment and one or more discharge locations to deliver material to the downstream equipment or receiving vessel. The Bio-SCREW is easily adapted to congested locations for horizontal, inclined, or vertical conveyance and is designed to meet unique and specific plant needs. The Bio-SCREW is also resistant to high temperature, abrasives, and sticky materials and is completely enclosed to prevent odors or spillage.

The Bio-SCREW features an emergency stop (E-Stop) switch with a pull cord that runs along the length of the Bio-SCREW. The pull cord may be manually activated to stop the Bio-SCREW and any interlocked pieces of equipment in the event of an emergency.

The Bio-SCREW also features a zero-speed switch that is used to detect stoppage of the Bio-SCREW, notify the control system, and stop any interlocked pieces of equipment.

During startup, the Bio-SCREW should run empty for a period of time as detailed herein prior to receiving material for processing and handling to avoid overloading or damaging the Bio-SCREW. During shutdown, the Bio-SCREW should continue to run long enough after all upstream feed equipment has been stopped such that all material is substantially cleared from the Bio-SCREW. When all material is substantially cleared, the Bio-SCREW can be shut down.

Please refer and adhere to any additional best practices for screw conveyors as detailed in all applicable CEMA standards, including ANSI/CEMA Standard No. 350 “Screw Conveyor for Bulk Materials” and ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual”.

## Initial Startup

When starting the equipment for the first time or after an extended shutdown, perform the following procedures. Note that an operator must be standing by the Bio-SCREW and associated control systems to stop the system in case of emergency.

- Visually inspect the Bio-SCREW and any associated equipment for damage. Notify JMS using the contact information provided herein if damage is observed.

- Verify that all preventative maintenance and lubrication procedures have been completed as directed in Section 6 - Maintenance, Lubrication, & Troubleshooting.

- Verify that the E-Stop switch flag arm is in the non-tripped position (i.e. NOT upright).

- Verify that all personnel are clear of the equipment and that all obstructions (including any tools and/or debris) have been removed from the Bio-SCREW. Do not operate the Bio-SCREW until all obstructions have been safely cleared.

- Verify that all covers, safety devices, and controls are installed and operating correctly.

- Verify that the Bio-SCREW discharge is clear.

- Prior to starting any upstream feed equipment, start the Bio-SCREW.

- Allow the Bio-SCREW to operate empty for a maximum of five minutes (or as otherwise directed by JMS during dry startup procedures).

- Observe the Bio-SCREW for the following:

  - Overheating of the bearings

  - Unusual noises

  - Gear reducer/motor oil leakage and/or overheating (surface temperature of the gear reducer/motor should be less than 104º F).

  - Rough operation

- If any of the abnormalities from previous Step 9 are noted, refer to the appropriate maintenance actions and/or troubleshooting procedures provided in Section 6 - Maintenance, Lubrication, & Troubleshooting.

## Startup

During startup, an operator must be standing by the Bio-SCREW and associated control system to stop the Bio-SCREW in case of emergency. Perform the following procedures:

- Visually inspect the Bio-SCREW and any associated equipment for damage. Notify JMS using the contact information provided herein if damage is observed.

- Verify that all preventative maintenance and lubrication procedures have been completed as directed in Section 6 - Maintenance, Lubrication, & Troubleshooting.

- Verify that the E-Stop switch flag arm is in the non-tripped position (i.e. NOT upright).

- Verify that all personnel are clear of the equipment and that all obstructions (including any tools and/or debris) have been removed from the Bio-SCREW. Do not operate the Bio-SCREW until all obstructions have been safely cleared.

- Verify that all covers, safety devices, and controls are installed and operating correctly.

- Verify that the Bio-SCREW discharge is clear.

- Prior to starting any upstream feed equipment, start the Bio-SCREW.

- Once the Bio-SCREW has reached the intended operating speed, gradually begin to feed material into the Bio-SCREW by starting upstream feed equipment.

  - The feed rate should start low and be gradually increased until the specified design capacity is reached.

  - Do not overload the Bio-SCREW or otherwise exceed the equipment speed, capacity, volume, material density, or rate of flow for which the Bio-SCREW and gearmotor were designed.

## Shutdown

When it becomes necessary to shut down the equipment, perform the following procedures. Please note that the following procedures are intended to be for normal shutdown only. During an emergency shutdown due to an alarm condition, proceed to the instructions included in Section 5.5 - Shutdown (Alarm Condition).

- Shut down all upstream feed equipment according to the appropriate standard operating procedures.

- Allow the Bio-SCREW to continue running until all material has been substantially discharged from the equipment. Please note that the Bio-SCREW is designed with a small gap between the bottom of the screw flighting and the trough such that a nominal amount of material may remain in the bottom of the trough even when the majority of material has been cleared from the Bio-SCREW.

  - In the event that system controls will not permit this operation with the control switch in the “AUTO” position, the control switch may be placed in the “MANUAL” position to allow for manual adjustment. Refer to Appendix E: Controls if controls have been provided by JMS.

- Stop the gearmotor for the Bio-SCREW.

- LOTO all power to the Bio-SCREW.

- Inspect the Bio-SCREW bolts and alignments, realigning and tightening as necessary.

- [Inspect the hanger bearings and re-center as required.]

- In the event of an expected extended shutdown (longer than 30 days), the following additional procedures should be performed:

- Ensure all foreign material is removed from the Bio-SCREW or manually remove as necessary. If the material is corrosive, hygroscopic, or tends to harden (or “set up)”, the Bio-SCREW trough should be thoroughly and completely cleaned to remove any material that may remain in the bottom of the trough upon shutdown even when the majority of material has been discharged.

- Rotate the Bio-SCREW one and a half (1.5) rotations every two weeks. Screw rotation can be performed by bumping the motor or disengaging the motor and rotating the screw by hand.

- Adhere to all procedures for equipment storage included in Section 3.3 - Storage and any additional component manufacturer recommendations detailed in Appendix A: Catalog Cuts. This includes but is not limited to proper lubrication and protection of the gearmotor.

## Shutdown (Alarm Condition)

The Bio-SCREW (and other equipment if interlocked via the control system; refer to Appendix E: Controls if controls have been provided by JMS) may be stopped at any time by activating the E-Stop safety device associated with the screw. If the E-Stop safety device was the source of the emergency shutdown, perform the following actions:

- Shut down all upstream feed equipment and any other interconnected equipment according to the appropriate standard operating procedures.

- Stop the motor for the Bio-SCREW by turning the control switch to the “OFF” position.

- LOTO all power to the Bio-SCREW.

- Identify and rectify the cause of the E-Stop alarm by visually checking the equipment. This may involve the removal of screw covers and/or material in the screw.

- Visually inspect the screws for damage and address as necessary according to the details provided in Section 6 - Maintenance, Lubrication, & Troubleshooting.

- Reset the switch flag arm to the non-tripped position (i.e. NOT upright) and replace any covers (if removed).

- At the control panel, select the “ALARM RESET” button.

- To restart the equipment, turn the control switch to the “ON” position (or “AUTO” position as appropriate). Return to the normal shutdown procedures included in Section 5.4 - Shutdown when appropriate.

The Bio-SCREW has been supplied with a zero-speed switch that shuts down power to the associated motor if motion is lost (and other equipment if interlocked via the control system; refer to Appendix E: Controls if controls have been provided by JMS). If the zero-speed switch was the source of the emergency shutdown, perform the following actions:

- Shut down all upstream feed equipment and any other interconnected equipment according to the appropriate standard operating procedures.

- Stop the motor for the Bio-SCREW by turning the control switch to the “OFF” position.

- LOTO all power to the Bio-SCREW.

- Identify and rectify the cause of the zero-speed switch alarm by visually checking the equipment. This may involve the removal of screw covers and/or material in the screw.

- Visually inspect the screws for damage and address as necessary according to the details provided in Section 6 - Maintenance, Lubrication, & Troubleshooting. Replace any covers removed previously.

- At the control panel, select the “ALARM RESET” button.

- To restart the equipment, turn the control switch to the “ON” position (or “AUTO” position as appropriate). Return to the normal shutdown procedures included in Section 5.4 - Shutdown when appropriate.

# Maintenance, Lubrication, & Troubleshooting

While no special tools are required for working with the Bio-SCREW, care should be taken to avoid cross-contamination between carbon steel and stainless-steel components when installing, maintaining, or otherwise working with the equipment or components. Please refer to the additional information about carbon steel contamination provided in Section 3.2.1 - Avoiding Cross-Contamination Between Materials.

Please note that not all individual components comprising the Bio-SCREW are crafted by JMS (as detailed on the Bill of Materials on the drawings). Although JMS has carefully developed this comprehensive O&M manual to summarize the recommended procedures for effectively maintaining the Bio-SCREW and its components as a whole, any and all maintenance, lubrication, and/or troubleshooting procedures from individual component manufacturers for their specific component should supplement or take precedence over the requirements listed herein. Manufacturer reference information, including that which is supplied in Appendix A: Catalog Cuts, should always be referenced during maintenance, lubrication, and troubleshooting procedures where available. If questions arise over the proper maintenance, lubrication, and/or troubleshooting procedures, please contact JMS using the contact information provided in this O&M manual.

Please refer and adhere to any additional best practices for screw conveyors as detailed in all applicable CEMA standards, including ANSI/CEMA Standard No. 350 “Screw Conveyor for Bulk Materials” and ANSI/CEMA Standard 352 “Screw Conveyor Safety Operation and Maintenance Manual”.

## Preventative Maintenance

Before any maintenance or inspection is performed, refer to ANSI/ASSP Z244.1-2016 for minimum safety requirements concerning energy sources.

Practice good housekeeping. Keep the area around the equipment clean and free of obstacles to provide easy access. Establish routine periodic inspections of the equipment to ensure continuous maximum operating performance.

A basic maintenance tool kit is all that is required to maintain this equipment. No special tools are necessary.

Table 2. Bio-SCREW Preventative Maintenance

| Component | Recommended Maintenance | Frequency |
| --- | --- | --- |
| General | Ensure no debris or material is collecting on the face of sealing surfaces or otherwise between the drive shaft and gearmotor. Remove material as required. | Daily |
|  | Inspect the Bio-SCREW, especially discharge and inlet locations, for material buildup. Remove as required. | Daily |
|  | Ensure all trough covers and guards (if any) are installed and functioning properly. | Daily (or as often as equipment is in operation) |
|  | Inspect Bio-SCREW and gearmotor for unusual noises or vibrations. | Daily |
|  | Inspect for material spillage at inlet(s) and outlet(s). Clean any spills as required. Find cause of spillage and correct. | Daily |
|  | Visually inspect shaft seals on either end of the screw shaft for any leakage. Refer to Section 6.4 - Troubleshooting if leaks are observed. | Daily |
|  | Visually inspect the shaft couplings along the screw shaft for evidence of excessive movement. If wear to the shaft coupling holes exceeds 1/8” or causes excessive play, replace the shaft couplings. | Quarterly |
|  | Visually inspect the cross holes at both ends of the screw shaft for evidence of elongation. | Quarterly |
|  | Inspect coupling bolts and other associated hardware at both ends of the screw for evidence of damage or looseness. | Quarterly |
|  | Visually inspect the screw shaft and trough along its entire length for evidence of damage. This may require opening protective covers. | Quarterly |
|  | Visually inspect screw flights for evidence of wear and/or an inappropriate wear pattern. This may require opening associated protective covers. | Quarterly |
| External Hardware | Inspect all bolts and anchors and secure any that are loose. Refer to the guidance provided in Section 4.1 - Before Installation regarding proper hardware tightening. | Quarterly |
| SEW Eurodrive Gear Reducer | Check oil level. Refer to Section 6.3 - Lubrication. | Every 3,000 operating hours or every 6 months, whichever is sooner |
|  | Drain and refill the oil according to the information provided in Section 6.3 - Lubrication. | Every 8,000 operating hours or every 3 years, whichever is sooner |
| [Motor | Relubricate the motor bearings according to the details provided in Section 6.3 - Lubrication. | Refer to the interval provided in Section 6.3 - Lubrication.] |
| Screw Roller Bearing | Lubricate the bearing according to the details provided in Section 6.3 - Lubrication. | Refer to the interval provided in Section 6.3 - Lubrication. |
| [Screw Hanger Bearings | [Lubricate the bearing according to the details provided in Section 6.3 - Lubrication. | Refer to the interval provided in Section 6.3 - Lubrication.] |
|  | Inspect bearings for wear. If wear exceeds 1/8”, replace the bearing. | Quarterly] |
| Packing Gland Seal | Relubricate the seal according to the details provided in Section 6.3 - Lubrication. | Refer to the interval provided in Section 6.3 - Lubrication. |

## Corrective Maintenance

There are no recommended corrective maintenance procedures for the Bio-SCREW.

## Lubrication

Please refer to Figure 2 through Figure 4 for details about gear reducer mounting position and locations of components relating to the lubrication system. Please note that while the diagram shown in Figure 2 is sourced from the SEW Eurodrive O&M manual and is representative of the gearmotor provided for the project, the pictures provided in Figure 3 and Figure 4 are meant to serve as examples only and may not reflect the exact gearmotor or hardware orientation provided for this project.

[INSERT PIC OF MOUNTING POSITION FROM SEW MANUAL]

Figure 2. SEW Eurodrive Mounting Position "XXX" and Location of Lubrication Components

OIL LEVEL PLUG

BREATHER VALVE (PROTECTIVE BAND REMOVED)

Figure 3. Oil Level Plug (Painted Red) and Breather Valve (Protective Band Removed) (FOR REFERENCE ONLY)

Figure 4. Example Sight Glass in Lieu of Oil Level Plug (If Applicable) (FOR REFERENCE ONLY)

A lubrication schedule for all Bio-SCREW components is included in Table 3. Only check or change the oil when the gearmotor is not in operation and has had sufficient time to cool down to maintain safety and ensure proper measurement. While checking the oil, ensure that a container is placed beneath the oil level plug to capture small amounts of oil that may leak out. When changing the oil, ensure that oil is contained and disposed of in a proper manner.

Table 3. Bio-SCREW Component Lubrication Requirements

| Component | Activity | Interval | Quantity | Recommended Grease |
| --- | --- | --- | --- | --- |
| SEW Eurodrive Gear Reducer | Check oil level. | Every 3,000 operating hours or every 6 months, whichever is sooner | Refill to an acceptable level according to Figure 5. | See requirements for oil change below. |
|  | Drain oil and refill to oil level plug or sight glass. Refer to previous Figure 3 and Figure 4. | Every 8,000 operating hours or every 3 years, whichever is sooner | X | Recommended oil is listed on the SEW Eurodrive gear unit nameplate; if a specific brand is not listed, utilize the SEW Eurodrive brand oil for the listed oil type. |
| [Motor | Drain oil and refill according to the manufacturer’s procedure included within Appendix A: Catalog Cuts. | Refer to the interval (if any) provided in the manufacturer’s information within Appendix A: Catalog Cuts | Refer to the quantity (if any) provided in the manufacturer’s information within Appendix A: Catalog Cuts. | Refer to the recommended grease type (if any) provided in the manufacturer’s information within Appendix A: Catalog Cuts.] |
| Screw Roller Bearing | Refill the automatic lubrication system (if provided) according to the information provided within Appendix A: Catalog Cuts or otherwise relubricate the roller bearing. | Every 3 months | Until grease just purges from the bearing | Lithium complex-base grease (or a grease which is compatible with NLGI #2 lithium complex base grease) |
| [Screw Hanger Bearing | Refill the automatic lubrication system (if provided) or otherwise refer the information provided within Appendix A: Catalog Cuts regarding any applicable lubrication requirements. | Refer to the interval (if regreasing is required) provided in the manufacturer’s information within Appendix A: Catalog Cuts. | Refer to the quantity (if regreasing is required) provided in the manufacturer’s information within Appendix A: Catalog Cuts. | Refer to the recommended grease type (if regreasing is required) provided in the manufacturer’s information within Appendix A: Catalog Cuts.] |
| Packing Gland Seal | If the seal has a grease fitting, replenish grease. Otherwise, no action needed. | As required based on operation, at least quarterly. | Until grease just purges from the fitting. | Grease compatible with plant processing requirements. |

When checking the oil at the SEW Eurodrive gear reducer, utilize Figure 5 to determine the proper oil level at the oil level plug. Figure 5 has been adapted from the SEW Eurodrive O&M manual. The oil level plug should be removed to properly check the oil.

If a sight glass has been provided in the place of an oil level plug, the oil level can be checked visually through the sight glass without removing the sight glass. An example of a sight glass in lieu of an oil level plug is shown in previous Figure 4.

Figure 5. SEW Eurodrive Diagram for Checking Oil Level

If the oil is too low at the SEW Eurodrive gear reducer during a routine oil level check, perform the following steps:

- Remove the breather valve from the breather bore. Refer to previous Figure 2.

- Remove the oil level plug. Refer to previous Figure 3. If a sight glass has been provided, the sight glass does not need to be removed. Refer to previous Figure 4.

- Add oil of the same type via the breather bore up to the lower edge of the oil level bore.

- Replace the breather valve and screw the valve tight to a torque of xxx lb-ft (xxx Nm).

- Replace the oil level plug and tighten to a torque of xxx lb-ft (xxx Nm).

[The motor bearings are sealed for life and therefore do not have any lubrication requirements.]

## Troubleshooting

Table 4. Bio-SCREW Troubleshooting

| Problem | Probable Cause | Solution |
| --- | --- | --- |
| Bio-SCREW is not operating | Bio-SCREW is not receiving a run signal | Check controls (refer to Appendix E: Controls if controls have been provided by JMS) and verify that a run signal is being transmitted to the Bio-SCREW. |
|  | The E-Stop switch flag arm is in the tripped position (i.e. upright) | Reset the E-Stop switch flag arm to the non-tripped position (i.e. NOT upright). |
|  | Issue with other permissive control | Check other permissive controls such as zero-speed switches for proper operation and positioning. |
|  | Bio-SCREW is not receiving power | Following all proper plant SOPs and LOTO procedures, verify that there is power at the motor. |
|  | Bio-SCREW is jammed | LOTO power to the Bio-SCREW and manually clear material jams from the screw. |
|  | Motor failure | Notify JMS using the contact information provided in this O&M for recommended procedures. |
| Bio-SCREW motor trips out on overload | Too much material in the Bio-SCREW | Inspect the Bio-SCREW for proper loading. Verify that the Bio-SCREW design capacities (including speed, capacity, volume, material density, and/or rate of flow) are not exceeded. Reduce loading as required upon next startup. The feed rate should start low and gradually increase. |
|  | Worn or failed bearing causing screw misalignment or allowing screw to contact the trough | Inspect bearings and screw alignment. Replace failed bearings as necessary. |
| Material leaks through the seal on the end of the screw | [Drainpipe is blocked | Investigate and clear drainpipe blockage.] |
|  | Seal is worn | Add two pumps of grease to the seal. Tighten the nuts on the follower one turn at a time in a cross-tightening sequence, wait one hour, and repeat the tightening procedure again if leakage is still observed. If leakage increases instead of decreases, the seal requires replacement. |
| Material is leaking through the bearing | End bearing failure | Replace the bearing. |
|  | Lack of lubrication at screw shaft end bearing | Ensure that all bearings are regularly lubricated according to the details provided in Section 6.3 - Lubrication. |
| Premature trough wear | High spot on trough as installed | Eliminate high spot by carefully manipulating the affected area of the trough until the dent has been removed and the screw no longer contacts the trough when rotated. |
|  | High spot on screw as installed | Eliminate high spot by grinding the screw flighting until the screw no longer contacts the trough when rotated. |
|  | Screw is bent | Replace or repair screw as needed. Notify JMS using the contact information provided herein if a replacement screw is needed. Repair may be performed by carefully removing the affected screw in its entirety, cutting out the bent portion of the screw, bracing the bent portion of the screw, realigning the bent portion of the screw, welding the entire screw back together, and replacing the corrected screw. |
| Holes at the shaft connection are elongated | Bio-SCREW is subjected to frequent jogging or start/stop overload shock | Avoid frequently starting and stopping the screws. Eliminate overload shock by not starting or initially feeding the Bio-HOPPER at full speed or design capacity. In the event of excessive elongation that causes the bolts to be extremely loose, contact JMS for guidance on whether full screw replacement is needed. |
| Unusual noises are observed from the Bio-SCREW | Worn or failed bearing causing screw misalignment or allowing screw to contact the trough | Inspect bearings and screw alignment. Replace failed bearings as necessary. |

# Policies

## Warranty

All JMS equipment and/or materials are of high quality and are manufactured in conformity with the best commercial practices. JMS guarantees all equipment and materials manufactured or supplied by JMS to be free from defects in material and workmanship at the time of startup for a period of XXXXX. JMS will furnish without charge but will not install replacements for such parts and materials JMS finds to be defective.

This guarantee does not apply to any equipment which has been improperly handled or stored, improperly lubricated, subjected to misuse or neglect, misapplied with reference to the Engineer's Specifications, altered or tampered with, and/or damaged by corrective work performed without JMS’ specific written consent. No allowances will be made for such corrective work done without such consent.

Deterioration by chemical action, improper maintenance, or normal wear does not constitute defects and are therefore not covered by this warranty. Additionally, consumable items are not covered by this warranty. All warranty claims must be submitted within ten (10) calendar days of discovery of defects or they will be deemed waived. The foregoing is in lieu of all other warranties, whether expressed or implied.

## Liability

It is expressly understood that JMS’ liability for JMS products is limited to the furnishing of such replacement parts, and that JMS will not be liable for any other expense, injury, loss, or damage, whether direct or consequential, including but not limited to loss of profits, production, or increased cost of operation arising in connection with the sale or use of, or inability to use, JMS equipment and/or materials for any purpose except as herein provided.

In the event a customer's order form states any liability terms which are additional to or different from these set forth herein, this form must be acknowledged in writing by JMS to accept any liability set forth in such form(s).

## Returns

All returns must be authorized by JMS, and no credit will be allowed for returned items that are damaged in transit, incomplete, or received in an unsatisfactory condition. All items that are not eligible for credit will be returned to customer via freight collect or discarded upon customer's instructions.

## Delivery

Delivery dates represent only JMS’ best estimate of the time required to complete the work and ship the material from JMS’ plant. All orders are accepted with the understanding that shipping dates are approximate and subject to change because of factory conditions, fires, strikes, material shortages, civil or military authority, acts of God, or other causes beyond JMS knowledge or control.

Back charges against JMS for alterations, shortages, damage repair, and/or discrepancies will be accepted only with written prior approval from a duly authorized representative of JMS.

## Thread Lubricant Information

All JMS bolts and anchors are stainless-steel material unless otherwise specified. Without an application of thread lubricant, bolting materials may become locked together. JMS strongly recommends the use of NSF-61 approved thread lubricant on all stainless-steel bolts and anchors.

## Standard Erecting Practices

All fabricated equipment manufactured by JMS is shop checked for accuracy before shipment and should not present any erection problems. Due to specific field conditions, a reasonable amount of fit-up is considered standard erection practice. The use of tools such as “come-a-longs”, jacks, drift pins, and reamers is to be expected.

Standard erecting practice as defined by the AISC “Code of Standard Practice”, Section 7.14, states the following:

“The correction of minor misfits by moderate amounts of reaming, grinding, welding, or cutting, and the drawing of elements into line with drift pins shall be considered to be normal erection operations. Errors that cannot be corrected using the foregoing means, or that require major changes in member or connection configuration, shall be promptly reported to the ODRD, the ODRC, and the fabricator, by the erector, to enable the responsible entity to either correct the error or approve the most efficient and economical method of correction to be used by others.”

If a problem is traceable to JMS fabrication or engineering, JMS will work with you to resolve this problem. Back charges for replacing or correcting materials require prior written approval of an authorized JMS representative. The back charges will only be considered with prompt, complete cost documentation based on standard direct field labor hours and materials. Profit, overhead, and any other charges will not be allowed. Payment may be refused for corrective work without prior approval.

Problems other than minor misfits as defined by AISC 303 should be communicated in writing to JMS using the contact information provided in this O&M manual via the project’s non-conformance reporting procedure.

Appendix A: Catalog Cuts

Appendix B: Equipment Drawings

Appendix C: Calculations

Appendix D: Startup Reports

PLACEHOLDER

Startup reports will be completed and made available as appropriate.

Appendix E: Controls

Appendix F: O&M Revision History

| Date | Rev. | Description of Change | Author |
| --- | --- | --- | --- |
| INSERT DATE MM/DD/YYYY | 0 | Initial release. | INSERT INITIALS |